Faculty of Engineering, Architecture and Science

Department of Mechanical, Industrial, and Mechatronics Engineering

| Program | Electrical Engineering |
| --- | --- |
| Course Number | COE718 |
| Course Title | Embedded Systems Design |
| Semester/Year | F2025 |
| Instructor | Dr. Arghavan Asad |

| Lab/Tutorial Report NO. | 2 |
| --- | --- |

| Report Title | Exploring Cortex-M3 Features for Performance Efficiency |
| --- | --- |

| Section No. | 07 |
| --- | --- |
| Submission Date | September 24, 2025 |
| Due Date | September 25, 2025 |

| Name | Student ID | Signature* |
| --- | --- | --- |
| Shai Mourad | 501170415 |  |

*By signing above you attest that you have contributed to this submission and confirm that all

work you have contributed to this submission is your own work. Any suspicion of copying or

plagiarism in this work will result in an investigation of Academic Misconduct and may result

in a “0” on the work, an “F” in the course, or possibly more severe penalties, as well as a

Disciplinary Notice on your academic record under the Student Code of Academic Conduct,

which can be found online at:

https://www.torontomu.ca/senate/policies/pol60.pdf.

## Table of Contents

Introduction	2

Objectives	2

Procedure	2

Output (Screenshots)	3

Data	8

Methodology and Implementation	8

Performance Measurement	9

Discussion	9

Conclusion	10

References	10

Appendix  (Code) 	10

#### Introduction

This lab implements Cortex-M3 features that improve performance. Bit Banding is for single-cycle bit access, Conditional Execution (s suffix instructions and IT blocks) reduce branch penalties, and the barrel shifter to complete multiplication/division with shifts. By toggling LEDs on the LPC1768 as well as measuring their execution times, all performance methods were tested. .

#### Objectives

The objective of “Lab #2” is to further explore the ARM Cortex-M3 architecture. Specifically, the performance efficiency features: bit banding, conditional execution, and the barrel shifter. Bit banding allows direct manipulation of bits in memory and peripherals. Conditional execution  (s-suffix instructions and IT blocks) is used to reduce branch penalties. Finally, the barrel shifter optimizes arithmetic operations. An LED application was used using three different methods: masking, function, and direct bit banding to measure execution times in “Debug mode” at different optimization levels

#### Procedure

- Created a new Keil µVision project for LPC1768 and added provided source files (bitband.c, bitband.h, cond_ex.c, Barrel_shifting.c).

- Configured project settings: C99 mode, default compiler v5, Simulator for Debug, DLL parameters for LPC1768.

- Enabled execution profiling (Debug → Show Times) and Performance Analyzer.

- Compiled at different optimization levels (-O0 to -O3) and measured execution times of LED toggle methods.

- Collected conditional execution disassembly at -O0 and -O3.

- Measured one delay_ms() call execution time.

- Demonstrated barrel shifter expression and displayed result via LCD/printf.

#### Output (Screenshots)

Figure 1 – Project Workspace: Project workspace in Keil uVision showing all required source and header files for the Lab 2 program.

Figure 2 – Debug Mode: Debug mode with program halted at main(), showing registers, disassembly, and xPSR flags.

Figure 3 – Execution Profiling: Execution profiling results comparing masking, function, and direct bit banding methods.

Figure 4 – Conditional Execution: Disassembly window showing use of IT blocks and/or SUBS instructions.

(At -O0, the compiler produced explicit branch instructions (BNE), while at -O3 it generated IT blocks (ITE, ADDEQ), confirming conditional execution.)

(O0 Run)

(O3 Run)

Figure 5 – Barrel Shifting: Example instruction (LSL, ASR)

#### Data

### Table 1 – Execution Times of LED Toggle Methods at Different Optimization Levels

| Method | -O0 (No Opt) | -O1 | -O2 | -O3 | Fastest Result |
| --- | --- | --- | --- | --- | --- |
| Masking (AND/OR ops) | 0.830us | 0.430us | 0.320us | 0.860us | O2 |
| Function Bit Banding | 0.590us | 0.590us | 0.590us | 1.140us | TIED: O0, O1, O2 |
| Direct Bit Banding | 0.360us | 0.320us | 0.320us | 0.320us | TIED: O1, O2, O3 |

### Table 2 – Performance Comparison (% Improvement)

| Comparison | -O0 | -O1 | -O2 | -O3 |
| --- | --- | --- | --- | --- |
| Direct vs Mask | 56.63% | 25.58% | 0.00% | 62.79% |
| Direct vs Function | 38.98% | 45.76% | 45.76% | 71.93% |
| Mask vs Function | 28.92% | -37.21% | -84.37% | -32.56% |

Improvement=Slower Time−Faster TimeSlower Time × 100%

→ Delay = 2.034s using the Performance Analyzer (the whole code is run).

#### Methodology and Implementation

Within this experiment, three different methods to toggle LEDs on the LPC1768 were used. These methods were masking, function-based bit banding, and direct bit banding. Each method controlled two LEDs from different ports.

Masking:

On the GPIO registers, this approach used bitwise AND/OR operations directly so it could set/clear bits.

Function-based Bit Banding:

A formula is used to find the alias address at run-time:

Alias Address = Base Alias + (Byte Offset × 32) + (Bit Number × 4)

Direct Bit Banding:

With this method, precomputed alias addresses are hardcoded into the program.

The barrel shifter was uses an expression of the form:

Result = 1000 + (500 × n).

Multiplication is used by left shifts and division is by right shifts.

#### Performance Measurement

Execution times for all of the methods used in the lab were measured by the “Profiling” tool in  Keil µVision’s Debug mode. “Show Times” was also enabled to show the elapsed times for each LED toggle function which were recorded across different optimization levels (O0 to O3).

The “Delay” function was also profiled separately in Debug mode to measure the total execution time of one delay cycle. This way, accurate comparisons can be made for the  performance overhead introduced by the LED toggling methods versus the delay routine.

#### Discussion

Direct Bit Banding was the best performing method when compared to Masking or Function. This method avoids masking operations as well as run-time address calculations which significantly reduces total instructions

Masking was a moderate performance. It is simpler to code but each toggle needs a each toggle required a read-modify-write cycle, which introduces delays.

Function-based Bit Banding  was the worst of the three at higher optimization levels since additional time was added during runs due to computations.

These observations are reflected in the data tables. For instance,-O1 direct bit banding improved performance by over 45% compared to the function-based approach.

The LCD displayed proper results for Barrel Shifts, proving the use of barrel shifting to optimize arithmetic operations into single-cycle instructions.

The Debug and Target versions had different uses: “Debug” was for performance where “Target” was designed for the physical LED observation.  One focuses on accuracy of measurement, whereas the other focuses on functionality. Based on the performance analyzer, the two were quite different. The “Debug” version collects precise timing data, whereas the “Target” version introduced more delays,  masking differences in LED toggle execution times.

#### Conclusion

This lab perfectly demonstrates how Cortex-M3 features improve code efficiency. Direct bit banding was the fastest when compared to masking and function call methods. Conditional execution optimized code execution as well as reduced branch penalties. Barrel shifting gave  single-cycle arithmetic optimizations. The “Debug” version was used to collect performance data, while the “Target” version was functional with  LED toggling and LCD output. To conclude, this lab provides practical experience with the advanced ARM Cortex-M3 performance features.

#### References

#### Toronto Metropolitan University. (n.d.). Exploring Cortex-M3 Features for Performance Efficiency [PDF]. Embedded Systems Design Website. https://www.ecb.torontomu.ca/%7Ecourses/coe718/F22-labs/lab2.pdf

#### Appendix (Code)

## bitband.c

#include "LPC17xx.h" #include <stdio.h> #include <stdbool.h> #include "GLCD.h" #include "LED.h" #include "KBD.h" #define __FI 1 /* Font index 16x24 */ #define __USE_LCD 0 /* Uncomment to use the LCD */ //------- ITM Stimulus Port definitions for printf ------------------- // #define ITM_Port8(n) (*((volatile unsigned char *)(0xE0000000+4*n))) #define ITM_Port16(n) (*((volatile unsigned short*)(0xE0000000+4*n))) #define ITM_Port32(n) (*((volatile unsigned long *)(0xE0000000+4*n))) #define DEMCR (*((volatile unsigned long *)(0xE000EDFC))) #define TRCENA 0x01000000 struct __FILE { int handle; }; FILE __stdout; FILE __stdin; int fputc(int ch, FILE *f) { if (DEMCR & TRCENA) { while (ITM_Port32(0) == 0); ITM_Port8(0) = ch; } return(ch); } volatile unsigned long msTicks = 0; // Variable for SysTick Interrupt // SysTick Handler (Interrupt Service Routine for the SysTick interrupt) void SysTick_Handler(void) { msTicks++; } // Initialize SysTick void SysTick_Initialize(unsigned long ticks) { SysTick->LOAD = (ticks & 0xFFFFFF) - 1; // Set reload register SysTick->VAL = 0; // Reset the SysTick counter value SysTick->CTRL = 0x07; // Enable SysTick IRQ and SysTick Timer } // Delay using SysTick timer void delay_ms(unsigned long delayTicks) { unsigned long currentTicks; currentTicks = msTicks; // Save current ticks while ((msTicks - currentTicks) < delayTicks); // Wait until required time passes } //------------------------------------------------------------------- // // Bit Band Macros used to calculate the alias address at run time #define ADDRESS(x) (*((volatile unsigned long *)(x))) #define BitBand(x, y) ADDRESS(((unsigned long)(x) & 0xF0000000) ¦ 0x02000000 ¦(((unsigned long)(x) & 0x000FFFFF) << 5) ¦ ((y) << 2)) volatile unsigned long * bit1; volatile unsigned long * bit2; #define FIOPIN_Bit28 (*((volatile unsigned long *)0x233806F0)) #define FIOPIN_Bit02 (*((volatile unsigned long *)0x23380A88)) #define BASE_DELAY 1000 #define BASE_ADD 500 int currentShift = 0; int shiftDirection = 1; char buffer[10]; char buffer1[10]; char barshift[15] = "1000 + 500x"; bool led_switch = false; // shared state //------------------------------------------------------------------- // // Separate Methods //------------------------------------------------------------------- // void Mask_Method(void) { if (led_switch) { LPC_GPIO1->FIOPIN &= ~(1UL << 28); LPC_GPIO2->FIOPIN &= ~(1UL << 2); led_switch = false; } else { LPC_GPIO1->FIOPIN ¦= (1UL << 28); LPC_GPIO2->FIOPIN ¦= (1UL << 2); led_switch = true; } // Barrel shifter demo: replace multiply with shift int result = BASE_DELAY + (BASE_ADD << currentShift); delay_ms(result); } void Function_Method(void) { bit1 = &BitBand(&LPC_GPIO1->FIOPIN, 28); bit2 = &BitBand(&LPC_GPIO2->FIOPIN, 2); *bit1 = led_switch ? 0 : 1; *bit2 = led_switch ? 0 : 1; led_switch = !led_switch; delay_ms(BASE_DELAY + (BASE_ADD * currentShift)); } void Direct_BitBand_Method(void) { if (led_switch) { FIOPIN_Bit28 = 0; FIOPIN_Bit02 = 0; led_switch = false; } else { FIOPIN_Bit28 = 1; FIOPIN_Bit02 = 1; led_switch = true; } delay_ms(BASE_DELAY + (BASE_ADD * currentShift)); } //------------------------------------------------------------------- // // Main //------------------------------------------------------------------- // int main(void) { uint32_t joystick_val; SysTick_Initialize(100000); LED_Init(); KBD_Init(); #ifdef __USE_LCD GLCD_Init(); /* Initialize graphical LCD (if enabled) */ GLCD_Clear(White); /* Clear graphical LCD display */ GLCD_SetBackColor(Blue); GLCD_SetTextColor(Yellow); GLCD_DisplayString(0, 0, __FI, " COE718 Demo "); GLCD_SetTextColor(White); GLCD_DisplayString(1, 0, __FI, " bitband.c "); GLCD_SetBackColor(White); GLCD_SetTextColor(Blue); GLCD_DisplayString(3, 0, __FI, "Delay (ms): "); GLCD_DisplayString(4, 0, __FI, "1000 + 500x0 "); char base_buffer[10]; snprintf(base_buffer, sizeof(base_buffer), "%d", BASE_DELAY); GLCD_DisplayString(3, 11, __FI, (unsigned char *)base_buffer); GLCD_DisplayString(6, 0, __FI, "Mode: "); #endif while (1) { printf("\nMask Method\n"); Mask_Method(); printf("Barrel Result = %d\n", BASE_DELAY + (BASE_ADD << currentShift)); printf("\nFunction Method\n"); Function_Method(); printf("Barrel Result = %d\n", BASE_DELAY + (BASE_ADD << currentShift)); printf("\nDirect Bit Banding Method\n"); Direct_BitBand_Method(); printf("Barrel Result = %d\n", BASE_DELAY + (BASE_ADD << currentShift)); } }

## bitband.h

//***************************************************************************** // +--+ // ¦ ++----+ // +-++ ¦ // ¦ ¦ // +-+--+ ¦ // ¦ +--+--+ // +----+ Copyright (c) 2011 Code Red Technologies Ltd. // // Header file containing C macros to provide bitbanding on Cortex-M3 MCU's // // Software License Agreement // // The software is owned by Code Red Technologies and/or its suppliers, and is // protected under applicable copyright laws. All rights are reserved. Any // use in violation of the foregoing restrictions may subject the user to criminal // sanctions under applicable laws, as well as to civil liability for the breach // of the terms and conditions of this license. // // THIS SOFTWARE IS PROVIDED "AS IS". NO WARRANTIES, WHETHER EXPRESS, IMPLIED // OR STATUTORY, INCLUDING, BUT NOT LIMITED TO, IMPLIED WARRANTIES OF // MERCHANTABILITY AND FITNESS FOR A PARTICULAR PURPOSE APPLY TO THIS SOFTWARE. // USE OF THIS SOFTWARE FOR COMMERCIAL DEVELOPMENT AND/OR EDUCATION IS SUBJECT // TO A CURRENT END USER LICENSE AGREEMENT (COMMERCIAL OR EDUCATIONAL) WITH // CODE RED TECHNOLOGIES LTD. // //***************************************************************************** #ifndef BITBAND_H_ #define BITBAND_H_ /* The Cortex-M3 memory map includes two bit-band regions. These occupy the lowest * 1MB of the SRAM and peripheral memory regions respectively. * + SRAM: Bit-band region: 0x20000000 - 0x20100000 * Bit-band alias: 0x22000000 - 0x23FFFFFF * + PERI: Bit-band region: 0x40000000 - 0x40100000 * Bit-band alias: 0x42000000 - 0x43FFFFFF * The mapping formula: * bit_word_offset = (byte_offset * 32) + (bit_number * 4) * bit_word_address = bit_band_base + bit_word_offset * where: * + bit_word_offset: the position of the target bit in the bit-band memory region * + bit_word_addr: the address of the word in the alias memory region that maps to the target bit * + bit_band_base: the starting address of the alias region * + byte_offset: the number of byte in the bit-band region that contains the targeted bit * + bit_number: is the bit position (0-7) of the targeted bit */ /* Bit band SRAM definitions */ #define BITBAND_SRAM_REF 0x20000000 #define BITBAND_SRAM_BASE 0x22000000 #define BITBAND_SRAM(a,b) ((BITBAND_SRAM_BASE + ((a-BITBAND_SRAM_REF)<<5) + (b<<2))) // Convert SRAM address /* Bit band PERIPHERAL definitions */ #define BITBAND_PERI_REF 0x40000000 #define BITBAND_PERI_BASE 0x42000000 #define BITBAND_PERI(a,b) ((BITBAND_PERI_BASE + ((a-BITBAND_PERI_REF)<<5) + (b<<2))) // Convert PERI address /* Basic bit band function definitions */ #define BITBAND_SRAM_ClearBit(a,b) (*(volatile uint32_t *) (BITBAND_SRAM(a,b)) = 0) #define BITBAND_SRAM_SetBit(a,b) (*(volatile uint32_t *) (BITBAND_SRAM(a,b)) = 1) #define BITBAND_SRAM_GetBit(a,b) (*(volatile uint32_t *) (BITBAND_SRAM(a,b))) #define BITBAND_PERI_ClearBit(a,b) (*(volatile uint32_t *) (BITBAND_PERI(a,b)) = 0) #define BITBAND_PERI_SetBit(a,b) (*(volatile uint32_t *) (BITBAND_PERI(a,b)) = 1) #define BITBAND_PERI_GetBit(a,b) (*(volatile uint32_t *) (BITBAND_PERI(a,b))) #endif /* BITBAND_H_ */

## cond_ex.c

//conditional execution code example #include "LPC17xx.h" int main(void){ int r1 = 1, r2 = 0; while(r1 <= 0x07){ if((r1 - r2) > 0){ r1 = r1 + 2; } else{ r2 = r2 + 1; } } return 0; }