Faculty of Engineering, Architecture and Science

Department of Mechanical, Industrial, and Mechatronics Engineering

| Program | Electrical Engineering |
| --- | --- |
| Course Number | COE718 |
| Course Title | Embedded Systems Design |
| Semester/Year | F2025 |
| Instructor | Dr. Arghavan Asad |

| Lab/Tutorial Report NO. | 3a |
| --- | --- |

| Report Title | RTX Based Multitasking with Round-Robin Scheduling |
| --- | --- |

| Section No. | 07 |
| --- | --- |
| Submission Date | October 8, 2025 |
| Due Date | October 9, 2025 |

| Name | Student ID | Signature* |
| --- | --- | --- |
| Shai Mourad | 501170415 |  |

*By signing above you attest that you have contributed to this submission and confirm that all

work you have contributed to this submission is your own work. Any suspicion of copying or

plagiarism in this work will result in an investigation of Academic Misconduct and may result

in a “0” on the work, an “F” in the course, or possibly more severe penalties, as well as a

Disciplinary Notice on your academic record under the Student Code of Academic Conduct,

which can be found online at:

https://www.torontomu.ca/senate/policies/pol60.pdf.

## Table of Contents

Introduction	2

Objectives	2

Procedure	2

Output (Screenshots)	3

Data	4

Methodology and Implementation	6

Performance Measurement	6

Discussion	6

Conclusion	6

References	7

Appendix  (Code) 	7

#### Introduction

The lab acts as an introduction to RTX, a RTOS to implement multitasking on the LPC1768 Cortex-M3 microcontroller. In contrast to previous labs, this experiment emphasizes round-robin scheduling across multiple threads. Using Kiel µVision tools such as the Watch Window, Performance Analyzer, and Event Viewer, observations of behavior for two and later three threads with round-robin scheduling is conducted.

#### Objectives

The main objectives of “Lab 3a” are to understand how RTX manages multiple threads at once using round-robin techniques. Also to configure kernel parameters including system tick frequency and time slice, analyze execution of multiple threads using “Debug” tools, implement finite workloads, and finally to compare the performance and CPU utilization of different task scheduling scenarios.

#### Procedure

- Created a new Keil µVision project "Lab3a_Demo" using the LPC1768 chip.

- Added the CMSIS > CORE, CMSIS > RTOS (API) > Keil RTX, and Device > Startup packages.

- Inserted template files for CMSIS-RTOS main.c and Threads.c, then replaced them with provided starter code.

- Configured project options: compiler v5, MicroLIB enabled, and simulator debug with DLL parameters (DARMP1.DLL, -pLPC1768).

- Configured RTX_Conf_CM.c using the Configuration Wizard:

  - SysTick timer as kernel timer.

  - Input clock = 10 MHz.

  - Tick interval = 10 ms.

  - Round-robin scheduling enabled.

- Built and ran the project in Debug mode.

- Used Watch Window to observe variables counta and countb.

- Opened Performance Analyzer to track thread activity.

- Used Event Viewer to observe 10 ms round-robin time slices.

- Modified RTX_Conf_CM.c to test different time slices and observed updated scheduling.

- Extended the demo to include three tasks with finite workloads for round-robin testing.

#### Output (Screenshots)

Figure 1 – Performance Analyzer and Event Viewer for Lab 3a (Part 1)

Figure 2 – Event Viewer Output for Lab 3a (Part 2)

#### Data

### Table 1 – Round-Robin Scheduling Parameters

| Parameter | Lab 3a Part 1 (2 Threads) | Lab 3a Part 2 (5 Threads) |
| --- | --- | --- |
| RTX Kernel Timer Clock (Hz) | 10 MHz | 10 MHz |
| Time-Slice Interval (µs) | 10 000 (10 ms) | 15 000 (15 ms) |
| Scheduler Type | Round Robin | Round Robin |
| Execution Mode | Simulator (Debug) | Simulator (Debug) |
| CPU Utilization (countIDLE) | ≈ 0 % (fully utilized) | ≈ 0 % (fully utilized) |

### Table 2 – Observed Thread Execution Sequence and Timing

| Thread Name | Function Type | Execution Pattern in Event Viewer | Average Slice Duration (ms) | Notes |
| --- | --- | --- | --- | --- |
| Thread 1 | Basic Counter / Toggle | Executes first after kernel start | ≈ 10 ms | Alternates with Thread 2 in Part 1 |
| Thread 2 | Secondary Counter / Toggle | Executes after Thread 1 | ≈ 10 ms | Alternates smoothly with Thread 1 |
| Thread 3 | Memory Management | Sequential after Thread 2 → Part 2 | ≈ 15 ms | Performs bit-band and signal ops |
| Thread 4 | CPU Management | Executes after Memory Mgmt | ≈ 15 ms | Includes barrel-shift test |
| Thread 5 | Application / Device / User Interface | Follows Thread 4 in rotation | ≈ 15 ms | Accesses global mutex and delays 1 tick |

### Table 3 – Variable Values from Watch Window (Part 2)

| Variable | Description | Value (Example Run) |
| --- | --- | --- |
| countIDLE | Idle task counter | 0x0010 (≈ no idle time) |
| memory_access_counter | Memory Mgmt thread counter | 0x00000001 |
| cpu_mgmt_access_counter | CPU Mgmt thread counter | 0x00000001 |
| app_interface_counter | App Interface thread counter | 0x00000001 |
| device_mgmt_counter | Device Mgmt thread counter | 0x00000001 |
| num_of_users | User Interface counter | 0x00000001 |

#### Methodology and Implementation

Two different versions were implemented:

- Analysis Version:

Focusing on round-robin task scheduling, this version uses simple counter variables (counta, countb, countc) while no LED/LCD functions included to keep execution purely computational.

- Demo Version:

Extended with LED and LCD output to visualize thread execution. Each thread updates LED and LCD outputs, while conditional delays were implemented to make the execution observable.

#### Performance Measurement

This lab uses the Watch Window, Performance Analyzer, Event Viewer, and Idle Task Measurement. These measuring tools ensure proper outputs of counters, executions of threads, 10ms and 15ms slices, as well as ensuring countIDLE remained 0, proving CPU was fully utilized.

#### Discussion

This lab demonstrates that RTX round-robin scheduling provides good allocation of CPU time across various tasks. Each thread is given equal execution time, and execution alternated as expected. When the experiment extends to three tasks, the fairness continues, although each retrieves a smaller share of CPU usage per cycle.

Advantages of this method include simplicity of implementation, equal CPU distribution without priority bias, and predictable timing when workload is balanced. There were however a few disadvantages such as no differentiation for higher priority tasks, inefficient for tasks with different execution requirements,or if a task has heavy computation, it may delay responsiveness.

For the OS section in the assignment (Memory Management, CPU Management, App Interface, etc.), round-robin ensured fairness, however dependencies between tasks required careful use of synchronization (signals and mutexes).

#### Conclusion

To conclude the lab experiment findings, observations demonstrated successful multitasking using the RTX real-time operating system. Round-robin scheduling was implemented and analyzed using the Watch Window, Performance Analyzer, and Event Viewer. Equal times slices verified the behavior of multiple threads while the limitations are discussed and observed. This experiment provided a thorough understanding of RTOS scheduling and its impact on embedded systems design.

#### References

Toronto Metropolitan University. (n.d.). Lab 3a: RTX Based Multitasking with Round-Robin Scheduling [PDF]. Embedded Systems Design Website.
 https://www.ecb.torontomu.ca/~courses/coe718/

#### Appendix (Code)

## main.c

/*---------------------------------------------------------------------------- * CMSIS-RTOS 'main' function template *---------------------------------------------------------------------------*/ #define osObjectsPublic // define objects in main module #include "osObjects.h" // RTOS object definitions #include "LED.h" #include "GLCD.h" //#define __DEMO 0 #define __FI 1 extern int Init_Thread (void); /* * main: initialize and start the system */ int main (void) { osKernelInitialize (); // initialize CMSIS-RTOS LED_Init(); #ifdef __DEMO GLCD_Init(); /* Initialize graphical LCD (if enabled */ GLCD_Clear(White); /* Clear graphical LCD display */ GLCD_SetBackColor(Blue); GLCD_SetTextColor(Yellow); GLCD_DisplayString(0, 0, __FI, (unsigned char *)" COE718 Demo "); GLCD_SetTextColor(White); GLCD_DisplayString(1, 0, __FI, (unsigned char *)" Lab3a Part 2 "); GLCD_SetBackColor(White); GLCD_SetTextColor(Blue); GLCD_DisplayString(6, 0, __FI, (unsigned char *)"Thread: "); #endif Init_Thread (); osKernelStart (); // start thread execution osDelay(osWaitForever); }

## thread.c

#include "cmsis_os.h" // CMSIS RTOS header file #include "osObjects.h" // RTOS object definitions #include <stdio.h> #include <string.h> #include "LPC17xx.h" #include "LED.h" #include "GLCD.h" #include <math.h> //#define __DEMO 0 #define __FI 1 // Bit Band Macros used to calculate the alias address at run time #define ADDRESS(x) (*((volatile unsigned long *)(x))) #define BitBand(x, y) ADDRESS(((unsigned long)(x) & 0xF0000000) ¦ 0x02000000 ¦(((unsigned long)(x) & 0x000FFFFF) << 5) ¦ ((y) << 2)) volatile unsigned long * bit; int a=0; int b=1; int c=0; /*---------------------------------------------------------------------------- * Global Variables *---------------------------------------------------------------------------*/ volatile unsigned int memory_access_counter = 0; volatile unsigned int cpu_mgmt_access_counter = 0; volatile unsigned int app_interface_counter = 0; volatile unsigned int device_mgmt_counter = 0; volatile unsigned int num_of_users = 0; char logger[100]; osMutexDef(LOGGER_MUTEX); osMutexId logger_mutex; /*---------------------------------------------------------------------------- * Sample threads *---------------------------------------------------------------------------*/ void delay(int time) { int x = 0; while (x++ < (time*10000)) {} } void Thread1 (void const *argument); // thread function void Thread2 (void const *argument); // thread function void Thread3 (void const *argument); // thread function void Thread4 (void const *argument); // thread function void Thread5 (void const *argument); // thread function osThreadId tid_Thread; // thread id osThreadDef (Thread1, osPriorityHigh, 1, 0); // thread object osThreadId tid2_Thread; // thread id osThreadDef (Thread2, osPriorityHigh, 1, 0); // thread object osThreadId tid3_Thread; // thread id osThreadDef (Thread3, osPriorityAboveNormal, 1, 0); // thread object osThreadId tid4_Thread; // thread id osThreadDef (Thread4, osPriorityAboveNormal, 1, 0); // thread object osThreadId tid5_Thread; // thread id osThreadDef (Thread5, osPriorityNormal, 1, 0); // thread object int Init_Thread (void) { logger_mutex = osMutexCreate(osMutex(LOGGER_MUTEX)); tid_Thread = osThreadCreate (osThread(Thread1), NULL); tid2_Thread = osThreadCreate (osThread(Thread2), NULL); tid3_Thread = osThreadCreate (osThread(Thread3), NULL); tid4_Thread = osThreadCreate (osThread(Thread4), NULL); tid5_Thread = osThreadCreate (osThread(Thread5), NULL); if(!tid_Thread) return(-1); if(!tid2_Thread) return(-1); if(!tid3_Thread) return(-1); if(!tid4_Thread) return(-1); if(!tid5_Thread) return(-1); return(0); } void Thread5 (void const *argument) { // User Interface #ifdef __DEMO LED_Off(3); GLCD_DisplayString(6, 0, __FI, (unsigned char *)"Thread: 5 "); LED_On(4); #endif num_of_users++; osDelay(1); osThreadTerminate(tid5_Thread); } void Thread4 (void const *argument) { // Device Management #ifdef __DEMO LED_Off(2); GLCD_DisplayString(6, 0, __FI, (unsigned char *)"Thread: 4 "); LED_On(3); #endif osSignalWait(0x03, osWaitForever); // Wait for signal from Application Interface strcat(logger, "Ending from Device Management."); osSignalSet(tid3_Thread, 0x04); // Signal Application Interface osThreadYield(); device_mgmt_counter++; osDelay(1); osThreadTerminate(tid4_Thread); } void Thread3 (void const *argument) { // Application Interface #ifdef __DEMO LED_Off(1); GLCD_DisplayString(6, 0, __FI, (unsigned char *)"Thread: 3 "); LED_On(2); #endif osMutexWait(logger_mutex, osWaitForever); sprintf(logger, "Partial message from Application Interface: "); osSignalSet(tid4_Thread, 0x03); // Signal Device Management osThreadYield(); osSignalWait(0x04, osWaitForever); // Wait for signal from Device Management app_interface_counter++; osDelay(1); osThreadTerminate(tid3_Thread); } void Thread2 (void const *argument) { // CPU Management #ifdef __DEMO LED_Off(0); GLCD_DisplayString(6, 0, __FI, (unsigned char *)"Thread: 2 "); LED_On(1); #endif cpu_mgmt_access_counter++; // Conditional Execution and barrel shifting scenario while (c <= 3) { if (a > 10) { a--; } else { a++; } b = a + (b*2); c++; } c = 0; osSignalSet(tid_Thread, 0x02); // Signal Memory Management osThreadTerminate(tid2_Thread); } void Thread1 (void const *argument) { // Memory Management #ifdef __DEMO LED_Off(4); GLCD_DisplayString(6, 0, __FI, (unsigned char *)"Thread: 1 "); LED_On(0); #endif memory_access_counter++; bit = &BitBand(&LPC_GPIO2->FIOPIN, 2); osThreadYield(); osSignalWait(0x02, osWaitForever); // Wait for signal from CPU Management osDelay(1); osThreadTerminate(tid_Thread); }

## RTX_Conf_CM.c

/*---------------------------------------------------------------------------- * CMSIS-RTOS - RTX *---------------------------------------------------------------------------- * Name: RTX_Conf_CM.C * Purpose: Configuration of CMSIS RTX Kernel for Cortex-M * Rev.: V4.70.1 *---------------------------------------------------------------------------- * * Copyright (c) 1999-2009 KEIL, 2009-2016 ARM Germany GmbH. All rights reserved. * * SPDX-License-Identifier: Apache-2.0 * * Licensed under the Apache License, Version 2.0 (the License); you may * not use this file except in compliance with the License. * You may obtain a copy of the License at * * www.apache.org/licenses/LICENSE-2.0 * * Unless required by applicable law or agreed to in writing, software * distributed under the License is distributed on an AS IS BASIS, WITHOUT * WARRANTIES OR CONDITIONS OF ANY KIND, either express or implied. * See the License for the specific language governing permissions and * limitations under the License. *---------------------------------------------------------------------------*/ #include "cmsis_os.h" unsigned int countIDLE = 0; /*---------------------------------------------------------------------------- * RTX User configuration part BEGIN *---------------------------------------------------------------------------*/ //-------- <<< Use Configuration Wizard in Context Menu >>> ----------------- // // <h>Thread Configuration // ======================= // // <o>Number of concurrent running user threads <1-250> // <i> Defines max. number of user threads that will run at the same time. // <i> Default: 6 #ifndef OS_TASKCNT #define OS_TASKCNT 6 #endif // <o>Default Thread stack size [bytes] <64-4096:8><#/4> // <i> Defines default stack size for threads with osThreadDef stacksz = 0 // <i> Default: 200 #ifndef OS_STKSIZE #define OS_STKSIZE 50 // this stack size value is in words #endif // <o>Main Thread stack size [bytes] <64-32768:8><#/4> // <i> Defines stack size for main thread. // <i> Default: 200 #ifndef OS_MAINSTKSIZE #define OS_MAINSTKSIZE 50 // this stack size value is in words #endif // <o>Number of threads with user-provided stack size <0-250> // <i> Defines the number of threads with user-provided stack size. // <i> Default: 0 #ifndef OS_PRIVCNT #define OS_PRIVCNT 0 #endif // <o>Total stack size [bytes] for threads with user-provided stack size <0-1048576:8><#/4> // <i> Defines the combined stack size for threads with user-provided stack size. // <i> Default: 0 #ifndef OS_PRIVSTKSIZE #define OS_PRIVSTKSIZE 0 // this stack size value is in words #endif // <q>Stack overflow checking // <i> Enable stack overflow checks at thread switch. // <i> Enabling this option increases slightly the execution time of a thread switch. #ifndef OS_STKCHECK #define OS_STKCHECK 1 #endif // <q>Stack usage watermark // <i> Initialize thread stack with watermark pattern for analyzing stack usage (current/maximum) in System and Thread Viewer. // <i> Enabling this option increases significantly the execution time of osThreadCreate. #ifndef OS_STKINIT #define OS_STKINIT 0 #endif // <o>Processor mode for thread execution // <0=> Unprivileged mode // <1=> Privileged mode // <i> Default: Privileged mode #ifndef OS_RUNPRIV #define OS_RUNPRIV 0 #endif // </h> // <h>RTX Kernel Timer Tick Configuration // ====================================== // <q> Use Cortex-M SysTick timer as RTX Kernel Timer // <i> Cortex-M processors provide in most cases a SysTick timer that can be used as // <i> as time-base for RTX. #ifndef OS_SYSTICK #define OS_SYSTICK 1 #endif // // <o>RTOS Kernel Timer input clock frequency [Hz] <1-1000000000> // <i> Defines the input frequency of the RTOS Kernel Timer. // <i> When the Cortex-M SysTick timer is used, the input clock // <i> is on most systems identical with the core clock. #ifndef OS_CLOCK #define OS_CLOCK 10000000 #endif // <o>RTX Timer tick interval value [us] <1-1000000> // <i> The RTX Timer tick interval value is used to calculate timeout values. // <i> When the Cortex-M SysTick timer is enabled, the value also configures the SysTick timer. // <i> Default: 1000 (1ms) #ifndef OS_TICK #define OS_TICK 15000 #endif // </h> // <h>System Configuration // ======================= // // <e>Round-Robin Thread switching // =============================== // // <i> Enables Round-Robin Thread switching. #ifndef OS_ROBIN #define OS_ROBIN 1 #endif // <o>Round-Robin Timeout [ticks] <1-1000> // <i> Defines how long a thread will execute before a thread switch. // <i> Default: 5 #ifndef OS_ROBINTOUT #define OS_ROBINTOUT 10 #endif // </e> // <e>User Timers // ============== // <i> Enables user Timers #ifndef OS_TIMERS #define OS_TIMERS 1 #endif // <o>Timer Thread Priority // <1=> Low // <2=> Below Normal <3=> Normal <4=> Above Normal // <5=> High // <6=> Realtime (highest) // <i> Defines priority for Timer Thread // <i> Default: High #ifndef OS_TIMERPRIO #define OS_TIMERPRIO 5 #endif // <o>Timer Thread stack size [bytes] <64-4096:8><#/4> // <i> Defines stack size for Timer thread. // <i> Default: 200 #ifndef OS_TIMERSTKSZ #define OS_TIMERSTKSZ 50 // this stack size value is in words #endif // <o>Timer Callback Queue size <1-32> // <i> Number of concurrent active timer callback functions. // <i> Default: 4 #ifndef OS_TIMERCBQS #define OS_TIMERCBQS 4 #endif // </e> // <o>ISR FIFO Queue size<4=> 4 entries <8=> 8 entries // <12=> 12 entries <16=> 16 entries // <24=> 24 entries <32=> 32 entries // <48=> 48 entries <64=> 64 entries // <96=> 96 entries // <i> ISR functions store requests to this buffer, // <i> when they are called from the interrupt handler. // <i> Default: 16 entries #ifndef OS_FIFOSZ #define OS_FIFOSZ 16 #endif // </h> //------------- <<< end of configuration section >>> ----------------------- // Standard library system mutexes // =============================== // Define max. number system mutexes that are used to protect // the arm standard runtime library. For microlib they are not used. #ifndef OS_MUTEXCNT #define OS_MUTEXCNT 8 #endif /*---------------------------------------------------------------------------- * RTX User configuration part END *---------------------------------------------------------------------------*/ #define OS_TRV ((uint32_t)(((double)OS_CLOCK*(double)OS_TICK)/1E6)-1) /*---------------------------------------------------------------------------- * Global Functions *---------------------------------------------------------------------------*/ /*--------------------------- os_idle_demon ---------------------------------*/ /// \brief The idle demon is running when no other thread is ready to run void os_idle_demon (void) { for (;;) { /* HERE: include optional user code to be executed when no thread runs.*/ countIDLE++; } } #if (OS_SYSTICK == 0) // Functions for alternative timer as RTX kernel timer /*--------------------------- os_tick_init ----------------------------------*/ /// \brief Initializes an alternative hardware timer as RTX kernel timer /// \return IRQ number of the alternative hardware timer int os_tick_init (void) { return (-1); /* Return IRQ number of timer (0..239) */ } /*--------------------------- os_tick_val -----------------------------------*/ /// \brief Get alternative hardware timer's current value (0 .. OS_TRV) /// \return Current value of the alternative hardware timer uint32_t os_tick_val (void) { return (0); } /*--------------------------- os_tick_ovf -----------------------------------*/ /// \brief Get alternative hardware timer's overflow flag /// \return Overflow flag\n /// - 1 : overflow /// - 0 : no overflow uint32_t os_tick_ovf (void) { return (0); } /*--------------------------- os_tick_irqack --------------------------------*/ /// \brief Acknowledge alternative hardware timer interrupt void os_tick_irqack (void) { /* ... */ } #endif // (OS_SYSTICK == 0) /*--------------------------- os_error --------------------------------------*/ /* OS Error Codes */ #define OS_ERROR_STACK_OVF 1 #define OS_ERROR_FIFO_OVF 2 #define OS_ERROR_MBX_OVF 3 #define OS_ERROR_TIMER_OVF 4 extern osThreadId svcThreadGetId (void); /// \brief Called when a runtime error is detected /// \param[in] error_code actual error code that has been detected void os_error (uint32_t error_code) { /* HERE: include optional code to be executed on runtime error. */ switch (error_code) { case OS_ERROR_STACK_OVF: /* Stack overflow detected for the currently running task. */ /* Thread can be identified by calling svcThreadGetId(). */ break; case OS_ERROR_FIFO_OVF: /* ISR FIFO Queue buffer overflow detected. */ break; case OS_ERROR_MBX_OVF: /* Mailbox overflow detected. */ break; case OS_ERROR_TIMER_OVF: /* User Timer Callback Queue overflow detected. */ break; default: break; } for (;;); } /*---------------------------------------------------------------------------- * RTX Configuration Functions *---------------------------------------------------------------------------*/ #include "RTX_CM_lib.h" /*---------------------------------------------------------------------------- * end of file *---------------------------------------------------------------------------*/