Faculty of Engineering, Architecture and Science

Department of Mechanical, Industrial, and Mechatronics Engineering

| Program | Electrical Engineering |
| --- | --- |
| Course Number | COE718 |
| Course Title | Embedded Systems Design |
| Semester/Year | F2025 |
| Instructor | Dr. Arghavan Asad |

| Lab/Tutorial Report NO. | 4 |
| --- | --- |

| Report Title | Real-time Scheduling and Priority Inversion Understanding |
| --- | --- |

| Section No. | 07 |
| --- | --- |
| Submission Date | October 29, 2025 |
| Due Date | October 30, 2025 |

| Name | Student ID | Signature* |
| --- | --- | --- |
| Shai Mourad | 501170415 |  |

*By signing above you attest that you have contributed to this submission and confirm that all

work you have contributed to this submission is your own work. Any suspicion of copying or

plagiarism in this work will result in an investigation of Academic Misconduct and may result

in a “0” on the work, an “F” in the course, or possibly more severe penalties, as well as a

Disciplinary Notice on your academic record under the Student Code of Academic Conduct,

which can be found online at:

https://www.torontomu.ca/senate/policies/pol60.pdf.

## Table of Contents

Introduction	2

Objectives	2

Procedure	2

Output (Screenshots)	3

Discussion	5

Conclusion	5

References	6

Appendix  (Code) 	6

#### Introduction

This lab has a focus on the use and analysis of ROTS on the ARM Cortex-M3 LPC1768 platform which expands on concepts from previous labs. This experiment introduces Rate Monotonic Scheduling (RMS). This is a fixed-priority technique and explores priority inversing and its mitigation. Using Keil µVision, virtual timers, thread signaling, and dynamic priority adjustments were employed to show real-time observations.

This experiment shows how fixed-priority scheduling gives deterministic task execution, and improper synchronization among threads leads to real-world scheduling problems such as the Mars Pathfinder priority inversion incident.

#### Objectives

This lab will allow for an understanding and implementation of RMS using RTX virtual timers and inter thread communications mechanisms. A study of priority inversion will be conducted to simulate samples, and then applying dynamic priority elevation for corrections. In addition, this lab will observe the effect of thread priorities, timing parameters, and synchronization on real-time performance using µVision’s Event Viewer and RTX Task Viewer. Overall this lab is designed to develop skills in making predictable, time-critical embedded applications that respect fixed-priority constraints.

#### Procedure

- Open the virtual_demo.c and priority_inv.c example projects and follow the lab manual instructions.

- In the Manage Run-Time Environment window, enable these packages:

  - CMSIS > CORE

  - CMSIS > RTOS (API) > Keil RTX

  - Device > Startup

- Alter virtual_demo.c to use an RMS algorithm using three processes as described in Part 1 of the lab manual. Then, edit priority_inv.c to use the Mars Pathfinder (MP) problem and its solution within a single C file.

- Build the project, then start the simulation using Debug.

- Run the project in Debug mode to analyze thread performance using the Performance Analyzer and Event Viewer.

#### Output (Screenshots)

Figure 1 - Debug Analysis Output for Part 1

Figure 2 - Event Viewer Output for Part 1

Figure 3  - Debug Analysis Output for Part 2

Figure 4 - Event Viewer Output for Part 2 (Solution to Mars Pathfinder)

#### Discussion

The results found in this lab clearly validate that RMS provides predictable, periodic task execution suitable for real-time embedded systems. The thread timing aligned with theory, showing the effectiveness of properly applied fixed-priority scheduling.

The priority inversion experiment introduced an important system flaw: high priority tasks can be indefinitely delayed by lower-priority threads when shared resources are not properly synchronized. Dynamic priority elevation effectively restored correct execution order.

This reflects real-world cases like the Mars Pathfinder incident, emphasizing the importance of mutex protocols and priority inheritance mechanisms in RTOS design.

#### Conclusion

This lab successfully uses both Rate Monotonic Scheduling and Priority Inversion Resolution on the LPC1768. By using RTX timers, inter-thread signals, and dynamic priority controls, real-time behavior was analyzed.

The experiments show the importance of RMS and how it time-bounded task completion under fixed priorities, while dynamic priority adjustment is essential to prevent priority inversion.

#### References

Toronto Metropolitan University. (n.d.). Real-time Scheduling and Priority Inversion (Lab 4). Embedded Systems Design Website.

https://www.ecb.torontomu.ca/~courses/coe718/F22-labs/lab4.pdf

#### Appendix (Code)

## virtual_demo.c (Q1)

;/**************************************************************************//** ; * @file startup_LPC17xx.s ; * @brief CMSIS Cortex-M3 Core Device Startup File for ; * NXP LPC17xx Device Series ; * @version V1.10 ; * @date 06. April 2011 ; * ; * @note ; * Copyright (C) 2009-2011 ARM Limited. All rights reserved. ; * ; * @par ; * ARM Limited (ARM) is supplying this software for use with Cortex-M ; * processor based microcontrollers. This file can be freely distributed ; * within development tools that are supporting such ARM based processors. ; * ; * @par ; * THIS SOFTWARE IS PROVIDED "AS IS". NO WARRANTIES, WHETHER EXPRESS, IMPLIED ; * OR STATUTORY, INCLUDING, BUT NOT LIMITED TO, IMPLIED WARRANTIES OF ; * MERCHANTABILITY AND FITNESS FOR A PARTICULAR PURPOSE APPLY TO THIS SOFTWARE. ; * ARM SHALL NOT, IN ANY CIRCUMSTANCES, BE LIABLE FOR SPECIAL, INCIDENTAL, OR ; * CONSEQUENTIAL DAMAGES, FOR ANY REASON WHATSOEVER. ; * ; ******************************************************************************/ ; *------- <<< Use Configuration Wizard in Context Menu >>> ------------------ ; <h> Stack Configuration ; <o> Stack Size (in Bytes) <0x0-0xFFFFFFFF:8> ; </h> Stack_Size EQU 0x00000200 AREA STACK, NOINIT, READWRITE, ALIGN=3 Stack_Mem SPACE Stack_Size __initial_sp ; <h> Heap Configuration ; <o> Heap Size (in Bytes) <0x0-0xFFFFFFFF:8> ; </h> Heap_Size EQU 0x00000000 AREA HEAP, NOINIT, READWRITE, ALIGN=3 __heap_base Heap_Mem SPACE Heap_Size __heap_limit PRESERVE8 THUMB ; Vector Table Mapped to Address 0 at Reset AREA RESET, DATA, READONLY EXPORT __Vectors __Vectors DCD __initial_sp ; Top of Stack DCD Reset_Handler ; Reset Handler DCD NMI_Handler ; NMI Handler DCD HardFault_Handler ; Hard Fault Handler DCD MemManage_Handler ; MPU Fault Handler DCD BusFault_Handler ; Bus Fault Handler DCD UsageFault_Handler ; Usage Fault Handler DCD 0 ; Reserved DCD 0 ; Reserved DCD 0 ; Reserved DCD 0 ; Reserved DCD SVC_Handler ; SVCall Handler DCD DebugMon_Handler ; Debug Monitor Handler DCD 0 ; Reserved DCD PendSV_Handler ; PendSV Handler DCD SysTick_Handler ; SysTick Handler ; External Interrupts DCD WDT_IRQHandler ; 16: Watchdog Timer DCD TIMER0_IRQHandler ; 17: Timer0 DCD TIMER1_IRQHandler ; 18: Timer1 DCD TIMER2_IRQHandler ; 19: Timer2 DCD TIMER3_IRQHandler ; 20: Timer3 DCD UART0_IRQHandler ; 21: UART0 DCD UART1_IRQHandler ; 22: UART1 DCD UART2_IRQHandler ; 23: UART2 DCD UART3_IRQHandler ; 24: UART3 DCD PWM1_IRQHandler ; 25: PWM1 DCD I2C0_IRQHandler ; 26: I2C0 DCD I2C1_IRQHandler ; 27: I2C1 DCD I2C2_IRQHandler ; 28: I2C2 DCD SPI_IRQHandler ; 29: SPI DCD SSP0_IRQHandler ; 30: SSP0 DCD SSP1_IRQHandler ; 31: SSP1 DCD PLL0_IRQHandler ; 32: PLL0 Lock (Main PLL) DCD RTC_IRQHandler ; 33: Real Time Clock DCD EINT0_IRQHandler ; 34: External Interrupt 0 DCD EINT1_IRQHandler ; 35: External Interrupt 1 DCD EINT2_IRQHandler ; 36: External Interrupt 2 DCD EINT3_IRQHandler ; 37: External Interrupt 3 DCD ADC_IRQHandler ; 38: A/D Converter DCD BOD_IRQHandler ; 39: Brown-Out Detect DCD USB_IRQHandler ; 40: USB DCD CAN_IRQHandler ; 41: CAN DCD DMA_IRQHandler ; 42: General Purpose DMA DCD I2S_IRQHandler ; 43: I2S DCD ENET_IRQHandler ; 44: Ethernet DCD RIT_IRQHandler ; 45: Repetitive Interrupt Timer DCD MCPWM_IRQHandler ; 46: Motor Control PWM DCD QEI_IRQHandler ; 47: Quadrature Encoder Interface DCD PLL1_IRQHandler ; 48: PLL1 Lock (USB PLL) DCD USBActivity_IRQHandler ; 49: USB Activity interrupt to wakeup DCD CANActivity_IRQHandler ; 50: CAN Activity interrupt to wakeup IF :LNOT::DEF:NO_CRP AREA ¦.ARM.__at_0x02FC¦, CODE, READONLY CRP_Key DCD 0xFFFFFFFF ENDIF AREA ¦.text¦, CODE, READONLY ; Reset Handler Reset_Handler PROC EXPORT Reset_Handler [WEAK] IMPORT SystemInit IMPORT __main LDR R0, =SystemInit BLX R0 LDR R0, =__main BX R0 ENDP ; Dummy Exception Handlers (infinite loops which can be modified) NMI_Handler PROC EXPORT NMI_Handler [WEAK] B . ENDP HardFault_Handler\ PROC EXPORT HardFault_Handler [WEAK] B . ENDP MemManage_Handler\ PROC EXPORT MemManage_Handler [WEAK] B . ENDP BusFault_Handler\ PROC EXPORT BusFault_Handler [WEAK] B . ENDP UsageFault_Handler\ PROC EXPORT UsageFault_Handler [WEAK] B . ENDP SVC_Handler PROC EXPORT SVC_Handler [WEAK] B . ENDP DebugMon_Handler\ PROC EXPORT DebugMon_Handler [WEAK] B . ENDP PendSV_Handler PROC EXPORT PendSV_Handler [WEAK] B . ENDP SysTick_Handler PROC EXPORT SysTick_Handler [WEAK] B . ENDP Default_Handler PROC EXPORT WDT_IRQHandler [WEAK] EXPORT TIMER0_IRQHandler [WEAK] EXPORT TIMER1_IRQHandler [WEAK] EXPORT TIMER2_IRQHandler [WEAK] EXPORT TIMER3_IRQHandler [WEAK] EXPORT UART0_IRQHandler [WEAK] EXPORT UART1_IRQHandler [WEAK] EXPORT UART2_IRQHandler [WEAK] EXPORT UART3_IRQHandler [WEAK] EXPORT PWM1_IRQHandler [WEAK] EXPORT I2C0_IRQHandler [WEAK] EXPORT I2C1_IRQHandler [WEAK] EXPORT I2C2_IRQHandler [WEAK] EXPORT SPI_IRQHandler [WEAK] EXPORT SSP0_IRQHandler [WEAK] EXPORT SSP1_IRQHandler [WEAK] EXPORT PLL0_IRQHandler [WEAK] EXPORT RTC_IRQHandler [WEAK] EXPORT EINT0_IRQHandler [WEAK] EXPORT EINT1_IRQHandler [WEAK] EXPORT EINT2_IRQHandler [WEAK] EXPORT EINT3_IRQHandler [WEAK] EXPORT ADC_IRQHandler [WEAK] EXPORT BOD_IRQHandler [WEAK] EXPORT USB_IRQHandler [WEAK] EXPORT CAN_IRQHandler [WEAK] EXPORT DMA_IRQHandler [WEAK] EXPORT I2S_IRQHandler [WEAK] EXPORT ENET_IRQHandler [WEAK] EXPORT RIT_IRQHandler [WEAK] EXPORT MCPWM_IRQHandler [WEAK] EXPORT QEI_IRQHandler [WEAK] EXPORT PLL1_IRQHandler [WEAK] EXPORT USBActivity_IRQHandler [WEAK] EXPORT CANActivity_IRQHandler [WEAK] WDT_IRQHandler TIMER0_IRQHandler TIMER1_IRQHandler TIMER2_IRQHandler TIMER3_IRQHandler UART0_IRQHandler UART1_IRQHandler UART2_IRQHandler UART3_IRQHandler PWM1_IRQHandler I2C0_IRQHandler I2C1_IRQHandler I2C2_IRQHandler SPI_IRQHandler SSP0_IRQHandler SSP1_IRQHandler PLL0_IRQHandler RTC_IRQHandler EINT0_IRQHandler EINT1_IRQHandler EINT2_IRQHandler EINT3_IRQHandler ADC_IRQHandler BOD_IRQHandler USB_IRQHandler CAN_IRQHandler DMA_IRQHandler I2S_IRQHandler ENET_IRQHandler RIT_IRQHandler MCPWM_IRQHandler QEI_IRQHandler PLL1_IRQHandler USBActivity_IRQHandler CANActivity_IRQHandler B . ENDP ALIGN ; User Initial Stack & Heap IF :DEF:__MICROLIB EXPORT __initial_sp EXPORT __heap_base EXPORT __heap_limit ELSE IMPORT __use_two_region_memory EXPORT __user_initial_stackheap __user_initial_stackheap LDR R0, = Heap_Mem LDR R1, =(Stack_Mem + Stack_Size) LDR R2, = (Heap_Mem + Heap_Size) LDR R3, = Stack_Mem BX LR ALIGN ENDIF END

## RTX_Conf_CM.c (Q1)

/*---------------------------------------------------------------------------- * CMSIS-RTOS - RTX *---------------------------------------------------------------------------- * Name: RTX_Conf_CM.C * Purpose: Configuration of CMSIS RTX Kernel for Cortex-M * Rev.: V4.70.1 *---------------------------------------------------------------------------- * * Copyright (c) 1999-2009 KEIL, 2009-2016 ARM Germany GmbH. All rights reserved. * * SPDX-License-Identifier: Apache-2.0 * * Licensed under the Apache License, Version 2.0 (the License); you may * not use this file except in compliance with the License. * You may obtain a copy of the License at * * www.apache.org/licenses/LICENSE-2.0 * * Unless required by applicable law or agreed to in writing, software * distributed under the License is distributed on an AS IS BASIS, WITHOUT * WARRANTIES OR CONDITIONS OF ANY KIND, either express or implied. * See the License for the specific language governing permissions and * limitations under the License. *---------------------------------------------------------------------------*/ #include "cmsis_os.h" /*---------------------------------------------------------------------------- * RTX User configuration part BEGIN *---------------------------------------------------------------------------*/ //-------- <<< Use Configuration Wizard in Context Menu >>> ----------------- // // <h>Thread Configuration // ======================= // // <o>Number of concurrent running user threads <1-250> // <i> Defines max. number of user threads that will run at the same time. // <i> Default: 6 #ifndef OS_TASKCNT #define OS_TASKCNT 6 #endif // <o>Default Thread stack size [bytes] <64-4096:8><#/4> // <i> Defines default stack size for threads with osThreadDef stacksz = 0 // <i> Default: 200 #ifndef OS_STKSIZE #define OS_STKSIZE 50 // this stack size value is in words #endif // <o>Main Thread stack size [bytes] <64-32768:8><#/4> // <i> Defines stack size for main thread. // <i> Default: 200 #ifndef OS_MAINSTKSIZE #define OS_MAINSTKSIZE 50 // this stack size value is in words #endif // <o>Number of threads with user-provided stack size <0-250> // <i> Defines the number of threads with user-provided stack size. // <i> Default: 0 #ifndef OS_PRIVCNT #define OS_PRIVCNT 0 #endif // <o>Total stack size [bytes] for threads with user-provided stack size <0-1048576:8><#/4> // <i> Defines the combined stack size for threads with user-provided stack size. // <i> Default: 0 #ifndef OS_PRIVSTKSIZE #define OS_PRIVSTKSIZE 0 // this stack size value is in words #endif // <q>Stack overflow checking // <i> Enable stack overflow checks at thread switch. // <i> Enabling this option increases slightly the execution time of a thread switch. #ifndef OS_STKCHECK #define OS_STKCHECK 1 #endif // <q>Stack usage watermark // <i> Initialize thread stack with watermark pattern for analyzing stack usage (current/maximum) in System and Thread Viewer. // <i> Enabling this option increases significantly the execution time of osThreadCreate. #ifndef OS_STKINIT #define OS_STKINIT 0 #endif // <o>Processor mode for thread execution // <0=> Unprivileged mode // <1=> Privileged mode // <i> Default: Privileged mode #ifndef OS_RUNPRIV #define OS_RUNPRIV 0 #endif // </h> // <h>RTX Kernel Timer Tick Configuration // ====================================== // <q> Use Cortex-M SysTick timer as RTX Kernel Timer // <i> Cortex-M processors provide in most cases a SysTick timer that can be used as // <i> as time-base for RTX. #ifndef OS_SYSTICK #define OS_SYSTICK 1 #endif // // <o>RTOS Kernel Timer input clock frequency [Hz] <1-1000000000> // <i> Defines the input frequency of the RTOS Kernel Timer. // <i> When the Cortex-M SysTick timer is used, the input clock // <i> is on most systems identical with the core clock. #ifndef OS_CLOCK #define OS_CLOCK 10000000 #endif // <o>RTX Timer tick interval value [us] <1-1000000> // <i> The RTX Timer tick interval value is used to calculate timeout values. // <i> When the Cortex-M SysTick timer is enabled, the value also configures the SysTick timer. // <i> Default: 1000 (1ms) #ifndef OS_TICK #define OS_TICK 10000 #endif // </h> // <h>System Configuration // ======================= // // <e>Round-Robin Thread switching // =============================== // // <i> Enables Round-Robin Thread switching. #ifndef OS_ROBIN #define OS_ROBIN 0 #endif // <o>Round-Robin Timeout [ticks] <1-1000> // <i> Defines how long a thread will execute before a thread switch. // <i> Default: 5 #ifndef OS_ROBINTOUT #define OS_ROBINTOUT 10 #endif // </e> // <e>User Timers // ============== // <i> Enables user Timers #ifndef OS_TIMERS #define OS_TIMERS 1 #endif // <o>Timer Thread Priority // <1=> Low // <2=> Below Normal <3=> Normal <4=> Above Normal // <5=> High // <6=> Realtime (highest) // <i> Defines priority for Timer Thread // <i> Default: High #ifndef OS_TIMERPRIO #define OS_TIMERPRIO 5 #endif // <o>Timer Thread stack size [bytes] <64-4096:8><#/4> // <i> Defines stack size for Timer thread. // <i> Default: 200 #ifndef OS_TIMERSTKSZ #define OS_TIMERSTKSZ 50 // this stack size value is in words #endif // <o>Timer Callback Queue size <1-32> // <i> Number of concurrent active timer callback functions. // <i> Default: 4 #ifndef OS_TIMERCBQS #define OS_TIMERCBQS 4 #endif // </e> // <o>ISR FIFO Queue size<4=> 4 entries <8=> 8 entries // <12=> 12 entries <16=> 16 entries // <24=> 24 entries <32=> 32 entries // <48=> 48 entries <64=> 64 entries // <96=> 96 entries // <i> ISR functions store requests to this buffer, // <i> when they are called from the interrupt handler. // <i> Default: 16 entries #ifndef OS_FIFOSZ #define OS_FIFOSZ 16 #endif // </h> //------------- <<< end of configuration section >>> ----------------------- // Standard library system mutexes // =============================== // Define max. number system mutexes that are used to protect // the arm standard runtime library. For microlib they are not used. #ifndef OS_MUTEXCNT #define OS_MUTEXCNT 8 #endif /*---------------------------------------------------------------------------- * RTX User configuration part END *---------------------------------------------------------------------------*/ #define OS_TRV ((uint32_t)(((double)OS_CLOCK*(double)OS_TICK)/1E6)-1) /*---------------------------------------------------------------------------- * Global Functions *---------------------------------------------------------------------------*/ /*--------------------------- os_idle_demon ---------------------------------*/ /// \brief The idle demon is running when no other thread is ready to run void os_idle_demon (void) { for (;;) { /* HERE: include optional user code to be executed when no thread runs.*/ } } #if (OS_SYSTICK == 0) // Functions for alternative timer as RTX kernel timer /*--------------------------- os_tick_init ----------------------------------*/ /// \brief Initializes an alternative hardware timer as RTX kernel timer /// \return IRQ number of the alternative hardware timer int os_tick_init (void) { return (-1); /* Return IRQ number of timer (0..239) */ } /*--------------------------- os_tick_val -----------------------------------*/ /// \brief Get alternative hardware timer's current value (0 .. OS_TRV) /// \return Current value of the alternative hardware timer uint32_t os_tick_val (void) { return (0); } /*--------------------------- os_tick_ovf -----------------------------------*/ /// \brief Get alternative hardware timer's overflow flag /// \return Overflow flag\n /// - 1 : overflow /// - 0 : no overflow uint32_t os_tick_ovf (void) { return (0); } /*--------------------------- os_tick_irqack --------------------------------*/ /// \brief Acknowledge alternative hardware timer interrupt void os_tick_irqack (void) { /* ... */ } #endif // (OS_SYSTICK == 0) /*--------------------------- os_error --------------------------------------*/ /* OS Error Codes */ #define OS_ERROR_STACK_OVF 1 #define OS_ERROR_FIFO_OVF 2 #define OS_ERROR_MBX_OVF 3 #define OS_ERROR_TIMER_OVF 4 extern osThreadId svcThreadGetId (void); /// \brief Called when a runtime error is detected /// \param[in] error_code actual error code that has been detected void os_error (uint32_t error_code) { /* HERE: include optional code to be executed on runtime error. */ switch (error_code) { case OS_ERROR_STACK_OVF: /* Stack overflow detected for the currently running task. */ /* Thread can be identified by calling svcThreadGetId(). */ break; case OS_ERROR_FIFO_OVF: /* ISR FIFO Queue buffer overflow detected. */ break; case OS_ERROR_MBX_OVF: /* Mailbox overflow detected. */ break; case OS_ERROR_TIMER_OVF: /* User Timer Callback Queue overflow detected. */ break; default: break; } for (;;); } /*---------------------------------------------------------------------------- * RTX Configuration Functions *---------------------------------------------------------------------------*/ #include "RTX_CM_lib.h" /*---------------------------------------------------------------------------- * end of file *---------------------------------------------------------------------------*/

## osObjects.h (Q2)

/*---------------------------------------------------------------------------- * osObjects.h: CMSIS-RTOS global object definitions for an application *---------------------------------------------------------------------------- * * This header file defines global RTOS objects used throughout a project * * #define osObjectsPublic indicates that objects are defined; without that * definition the objects are defined as external symbols. * *--------------------------------------------------------------------------*/ #ifndef __osObjects #define __osObjects #if (!defined (osObjectsPublic)) #define osObjectsExternal // define RTOS objects with extern attribute #endif #include "cmsis_os.h" // CMSIS RTOS header file // global 'thread' functions --------------------------------------------------- /* Example: extern void sample_name (void const *argument); // thread function osThreadId tid_sample_name; // thread id osThreadDef (sample_name, osPriorityNormal, 1, 0); // thread object */ // global 'semaphores' ---------------------------------------------------------- /* Example: osSemaphoreId sid_sample_name; // semaphore id osSemaphoreDef (sample_name); // semaphore object */ // global 'memory pools' -------------------------------------------------------- /* Example: typedef struct sample_name type_sample_name; // object data type osPoolId mpid_sample_name; // memory pool id osPoolDef (sample_name, 16, type_sample_name); // memory pool object */ // global 'message queues' ------------------------------------------------------- /* Example: typedef struct sample_name type_sample_name; // object data type osMessageQId mid_sample_name; // message queue id osMessageQDef (sample_name, 16, type_sample_name); // message queue object */ // global 'mail queues' ---------------------------------------------------------- /* Example: typedef struct sample_name type_sample_name; // object data type osMailQId qid_sample_name; // mail queue id osMailQDef (sample_name, 16, type_sample_name); // mail queue object */ #endif // __osObjects

## priority_inv.c (Q2)

/*---------------------------------------------------------------------------- * CMSIS-RTOS 'main' function template *---------------------------------------------------------------------------*/ #define osObjectsPublic // define objects in main module #include "osObjects.h" // RTOS object definitions #include "cmsis_os.h" // CMSIS RTOS header file #include <stdio.h> #include <math.h> #include "RTE_Components.h" // Component selection int a, b, c; /*---------------------------------------------------------------------------- CMSIS RTX Priority Inversion Example Priority Inversion = leave commented lines commented Priority Elevation = uncomment the 2 commented lines Anita Tino *----------------------------------------------------------------------------*/ void P1 (void const *argument); void P2 (void const *argument); void P3 (void const *argument); osThreadDef(P1, osPriorityHigh, 1, 0); osThreadDef(P2, osPriorityNormal, 1, 0); osThreadDef(P3, osPriorityBelowNormal, 1, 0); osThreadId t_main,t_P1,t_P2,t_P3; void delay(){ long k, count = 0; for(k = 0; k < 100000; k++){ count++; } } void P1 (void const *argument) { for (;;) { for (a = 0 ; a < 10; a++) { delay(); } osThreadSetPriority(t_P3, osPriorityHigh); //**solution uncomment osSignalSet(t_P3,0x01); //call P3 to finish the task osSignalWait(0x02,osWaitForever); //Error => priority inversion, P2 will run instead osThreadSetPriority(t_P3,osPriorityBelowNormal); //**solution uncomment } } void P2 (void const *argument) { for (;;) { for (b = 0 ; b < 20; b++) { delay(); } } } void P3 (void const *argument) { for (;;) { for (c = 0; c < 10 ; c++){ delay(); //do something } osSignalWait(0x01,osWaitForever); osSignalSet(t_P1,0x02); } } int main(void) { osKernelInitialize (); t_main = osThreadGetId (); osThreadSetPriority(t_main,osPriorityHigh); t_P3 = osThreadCreate(osThread(P3), NULL); osDelay(500); t_P2 = osThreadCreate(osThread(P2), NULL); osDelay(100); t_P1 = osThreadCreate(osThread(P1), NULL); osThreadTerminate(t_main); osKernelStart (); for (;;) {} }

## RTX_Conf_CM.c (Q2)

/*---------------------------------------------------------------------------- * CMSIS-RTOS - RTX *---------------------------------------------------------------------------- * Name: RTX_Conf_CM.C * Purpose: Configuration of CMSIS RTX Kernel for Cortex-M * Rev.: V4.70.1 *---------------------------------------------------------------------------- * * Copyright (c) 1999-2009 KEIL, 2009-2016 ARM Germany GmbH. All rights reserved. * * SPDX-License-Identifier: Apache-2.0 * * Licensed under the Apache License, Version 2.0 (the License); you may * not use this file except in compliance with the License. * You may obtain a copy of the License at * * www.apache.org/licenses/LICENSE-2.0 * * Unless required by applicable law or agreed to in writing, software * distributed under the License is distributed on an AS IS BASIS, WITHOUT * WARRANTIES OR CONDITIONS OF ANY KIND, either express or implied. * See the License for the specific language governing permissions and * limitations under the License. *---------------------------------------------------------------------------*/ #include "cmsis_os.h" /*---------------------------------------------------------------------------- * RTX User configuration part BEGIN *---------------------------------------------------------------------------*/ //-------- <<< Use Configuration Wizard in Context Menu >>> ----------------- // // <h>Thread Configuration // ======================= // // <o>Number of concurrent running user threads <1-250> // <i> Defines max. number of user threads that will run at the same time. // <i> Default: 6 #ifndef OS_TASKCNT #define OS_TASKCNT 6 #endif // <o>Default Thread stack size [bytes] <64-4096:8><#/4> // <i> Defines default stack size for threads with osThreadDef stacksz = 0 // <i> Default: 200 #ifndef OS_STKSIZE #define OS_STKSIZE 50 // this stack size value is in words #endif // <o>Main Thread stack size [bytes] <64-32768:8><#/4> // <i> Defines stack size for main thread. // <i> Default: 200 #ifndef OS_MAINSTKSIZE #define OS_MAINSTKSIZE 50 // this stack size value is in words #endif // <o>Number of threads with user-provided stack size <0-250> // <i> Defines the number of threads with user-provided stack size. // <i> Default: 0 #ifndef OS_PRIVCNT #define OS_PRIVCNT 0 #endif // <o>Total stack size [bytes] for threads with user-provided stack size <0-1048576:8><#/4> // <i> Defines the combined stack size for threads with user-provided stack size. // <i> Default: 0 #ifndef OS_PRIVSTKSIZE #define OS_PRIVSTKSIZE 0 // this stack size value is in words #endif // <q>Stack overflow checking // <i> Enable stack overflow checks at thread switch. // <i> Enabling this option increases slightly the execution time of a thread switch. #ifndef OS_STKCHECK #define OS_STKCHECK 1 #endif // <q>Stack usage watermark // <i> Initialize thread stack with watermark pattern for analyzing stack usage (current/maximum) in System and Thread Viewer. // <i> Enabling this option increases significantly the execution time of osThreadCreate. #ifndef OS_STKINIT #define OS_STKINIT 0 #endif // <o>Processor mode for thread execution // <0=> Unprivileged mode // <1=> Privileged mode // <i> Default: Privileged mode #ifndef OS_RUNPRIV #define OS_RUNPRIV 0 #endif // </h> // <h>RTX Kernel Timer Tick Configuration // ====================================== // <q> Use Cortex-M SysTick timer as RTX Kernel Timer // <i> Cortex-M processors provide in most cases a SysTick timer that can be used as // <i> as time-base for RTX. #ifndef OS_SYSTICK #define OS_SYSTICK 1 #endif // // <o>RTOS Kernel Timer input clock frequency [Hz] <1-1000000000> // <i> Defines the input frequency of the RTOS Kernel Timer. // <i> When the Cortex-M SysTick timer is used, the input clock // <i> is on most systems identical with the core clock. #ifndef OS_CLOCK #define OS_CLOCK 10000000 #endif // <o>RTX Timer tick interval value [us] <1-1000000> // <i> The RTX Timer tick interval value is used to calculate timeout values. // <i> When the Cortex-M SysTick timer is enabled, the value also configures the SysTick timer. // <i> Default: 1000 (1ms) #ifndef OS_TICK #define OS_TICK 10000 #endif // </h> // <h>System Configuration // ======================= // // <e>Round-Robin Thread switching // =============================== // // <i> Enables Round-Robin Thread switching. #ifndef OS_ROBIN #define OS_ROBIN 1 #endif // <o>Round-Robin Timeout [ticks] <1-1000> // <i> Defines how long a thread will execute before a thread switch. // <i> Default: 5 #ifndef OS_ROBINTOUT #define OS_ROBINTOUT 10 #endif // </e> // <e>User Timers // ============== // <i> Enables user Timers #ifndef OS_TIMERS #define OS_TIMERS 1 #endif // <o>Timer Thread Priority // <1=> Low // <2=> Below Normal <3=> Normal <4=> Above Normal // <5=> High // <6=> Realtime (highest) // <i> Defines priority for Timer Thread // <i> Default: High #ifndef OS_TIMERPRIO #define OS_TIMERPRIO 5 #endif // <o>Timer Thread stack size [bytes] <64-4096:8><#/4> // <i> Defines stack size for Timer thread. // <i> Default: 200 #ifndef OS_TIMERSTKSZ #define OS_TIMERSTKSZ 50 // this stack size value is in words #endif // <o>Timer Callback Queue size <1-32> // <i> Number of concurrent active timer callback functions. // <i> Default: 4 #ifndef OS_TIMERCBQS #define OS_TIMERCBQS 4 #endif // </e> // <o>ISR FIFO Queue size<4=> 4 entries <8=> 8 entries // <12=> 12 entries <16=> 16 entries // <24=> 24 entries <32=> 32 entries // <48=> 48 entries <64=> 64 entries // <96=> 96 entries // <i> ISR functions store requests to this buffer, // <i> when they are called from the interrupt handler. // <i> Default: 16 entries #ifndef OS_FIFOSZ #define OS_FIFOSZ 16 #endif // </h> //------------- <<< end of configuration section >>> ----------------------- // Standard library system mutexes // =============================== // Define max. number system mutexes that are used to protect // the arm standard runtime library. For microlib they are not used. #ifndef OS_MUTEXCNT #define OS_MUTEXCNT 8 #endif /*---------------------------------------------------------------------------- * RTX User configuration part END *---------------------------------------------------------------------------*/ #define OS_TRV ((uint32_t)(((double)OS_CLOCK*(double)OS_TICK)/1E6)-1) /*---------------------------------------------------------------------------- * Global Functions *---------------------------------------------------------------------------*/ /*--------------------------- os_idle_demon ---------------------------------*/ /// \brief The idle demon is running when no other thread is ready to run void os_idle_demon (void) { for (;;) { /* HERE: include optional user code to be executed when no thread runs.*/ } } #if (OS_SYSTICK == 0) // Functions for alternative timer as RTX kernel timer /*--------------------------- os_tick_init ----------------------------------*/ /// \brief Initializes an alternative hardware timer as RTX kernel timer /// \return IRQ number of the alternative hardware timer int os_tick_init (void) { return (-1); /* Return IRQ number of timer (0..239) */ } /*--------------------------- os_tick_val -----------------------------------*/ /// \brief Get alternative hardware timer's current value (0 .. OS_TRV) /// \return Current value of the alternative hardware timer uint32_t os_tick_val (void) { return (0); } /*--------------------------- os_tick_ovf -----------------------------------*/ /// \brief Get alternative hardware timer's overflow flag /// \return Overflow flag\n /// - 1 : overflow /// - 0 : no overflow uint32_t os_tick_ovf (void) { return (0); } /*--------------------------- os_tick_irqack --------------------------------*/ /// \brief Acknowledge alternative hardware timer interrupt void os_tick_irqack (void) { /* ... */ } #endif // (OS_SYSTICK == 0) /*--------------------------- os_error --------------------------------------*/ /* OS Error Codes */ #define OS_ERROR_STACK_OVF 1 #define OS_ERROR_FIFO_OVF 2 #define OS_ERROR_MBX_OVF 3 #define OS_ERROR_TIMER_OVF 4 extern osThreadId svcThreadGetId (void); /// \brief Called when a runtime error is detected /// \param[in] error_code actual error code that has been detected void os_error (uint32_t error_code) { /* HERE: include optional code to be executed on runtime error. */ switch (error_code) { case OS_ERROR_STACK_OVF: /* Stack overflow detected for the currently running task. */ /* Thread can be identified by calling svcThreadGetId(). */ break; case OS_ERROR_FIFO_OVF: /* ISR FIFO Queue buffer overflow detected. */ break; case OS_ERROR_MBX_OVF: /* Mailbox overflow detected. */ break; case OS_ERROR_TIMER_OVF: /* User Timer Callback Queue overflow detected. */ break; default: break; } for (;;); } /*---------------------------------------------------------------------------- * RTX Configuration Functions *---------------------------------------------------------------------------*/ #include "RTX_CM_lib.h" /*---------------------------------------------------------------------------- * end of file *---------------------------------------------------------------------------*/